Advanced SoC Lab-fsic_sim

111061545 陳揚哲

- Show the code that you use to program configuration address [‘h3000_5000].

There are multiple steps in every task: First, it will reset the system. Second, the system is initialized. Third, it enables the RX and TX side in system.

RX

TX

initialize

reset

From the step which is enable the RX and TX side in system, we can find out the how the system programs the specific address.

Therefore, I add new task changing the target address to [‘h3000_5000].

- Explain why “By programming configuration address [‘h3000_5000], signal user_prj_sel[4:0] will change accordingly”? (Hint: trace code in config_ctrl.v)

From these code, we change the user_prj_sel[4:0] signal if

(address[11:0] == 12’h000 && axi_wstrb[0] ==1). In the case, the program data would be the user_prj_sel signal.

- Briefly describe how you do FIR initialization (tap parameter, length) from SOC side (Test#1).

I use the soc_up_cfg_read and soc_up_cfg_write task to perform FIR initialization, since these tasks are programming the address base of 32’h3000_0000. First, I will check idle signal, then program the data length and taps coefficient and read back.

- Briefly describe how you do FIR initialization (tap parameter, length) from FPGA side (Test#2).

Similar to test#1, I use the fpga_axilite_read_req and fpga_axilite_write_req task to perform FIR initialization, since these tasks are programming the address base of 28’h0000_0000. First, I will check idle signal, then program the data length and taps coefficient and read back.

- Briefly describe how you feed in X data from FPGA side.

In order to feed in the X data, I modify the code of test002_fpga_axis_req, which is the test of axis. I only need to change the test002_fpga_axis_req, rather than the task it used. The task fpga_axis_req will config fpga_as signal (AXIS-Switch).

- Briefly describe how you get output Y data in testbench, and how to do comparison with golden values.

I read back the output data Y by following code, which is the original code I didn’t do any change. These code will read out and save the output in soc_to_fpga_axis_captured[soc_to_fpga_axis_captured_count][31:0].

- Screenshot simulation results printed on screen, to show that your Test#1 Test#2 complete successfully

Test#1:

Test#2

- Screenshot simulation waveform:

- Configuration cycle (when we program [‘h3000_5000] = 32’h01, signal user_prj_sel changes accordingly)

- AXI-Lite transaction cycles (feed in tap parameters, data_length)

Write the data length and tap parameters

From SOC side:

From FPGA side:

Read back the data length and tap parameters

From SOC side:

From FPGA side:

- Stream-in, Stream-out

There are several threads like the waveform below, until stream-in data reach 64.

The stream-out waveform shows the FIR will need several cycle to load new data and reset the previous output data.